BAND-E-KARKHEH

PRODUCTION FACILITIES BASIC DESIGN

PHASE I

PROCESS PLANT IDENTIFICATION & NUMBERING SYSTEM

| B4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| B3 |  |  |  |  |  |
| B2 |  |  |  |  |  |
| B1 |  |  |  |  |  |
| B0 | 28-JAN-2012 | ISSUED FOR REVIEW | A.N | R.A | N.KH |
| Rev. | DATE OF ISSUE | Purpose of Issue | Prepared | Checked | Approved |

Revision Sheet

| Rev. PAGE No. | B0 | B1 | B2 | B3 | B4 | Rev. PAGE No. | B0 | B1 | B2 | B3 | B4 | Rev. PAGE No. | B0 | B1 | B2 | B3 | B4 | B4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | X |  |  |  |  | 39 |  |  |  |  |  | 77 |  |  |  |  |  |  |
| 02 | X |  |  |  |  | 40 |  |  |  |  |  | 78 |  |  |  |  |  |  |
| 03 | X |  |  |  |  | 41 |  |  |  |  |  | 79 |  |  |  |  |  |  |
| 04 | X |  |  |  |  | 42 |  |  |  |  |  | 80 |  |  |  |  |  |  |
| 05 | X |  |  |  |  | 43 |  |  |  |  |  | 81 |  |  |  |  |  |  |
| 06 | X |  |  |  |  | 44 |  |  |  |  |  | 82 |  |  |  |  |  |  |
| 07 | X |  |  |  |  | 45 |  |  |  |  |  | 83 |  |  |  |  |  |  |
| 08 | X |  |  |  |  | 46 |  |  |  |  |  | 84 |  |  |  |  |  |  |
| 09 | X |  |  |  |  | 47 |  |  |  |  |  | 85 |  |  |  |  |  |  |
| 10 | X |  |  |  |  | 48 |  |  |  |  |  | 86 |  |  |  |  |  |  |
| 11 | X |  |  |  |  | 49 |  |  |  |  |  | 87 |  |  |  |  |  |  |
| 12 | X |  |  |  |  | 50 |  |  |  |  |  | 88 |  |  |  |  |  |  |
| 13 | X |  |  |  |  | 51 |  |  |  |  |  | 89 |  |  |  |  |  |  |
| 14 | X |  |  |  |  | 52 |  |  |  |  |  | 78 |  |  |  |  |  |  |
| 15 | X |  |  |  |  | 53 |  |  |  |  |  | 79 |  |  |  |  |  |  |
| 16 | X |  |  |  |  | 54 |  |  |  |  |  | 80 |  |  |  |  |  |  |
| 17 | X |  |  |  |  | 55 |  |  |  |  |  | 81 |  |  |  |  |  |  |
| 18 | X |  |  |  |  | 56 |  |  |  |  |  | 82 |  |  |  |  |  |  |
| 19 | X |  |  |  |  | 57 |  |  |  |  |  | 83 |  |  |  |  |  |  |
| 20 | X |  |  |  |  | 58 |  |  |  |  |  | 84 |  |  |  |  |  |  |
| 21 |  |  |  |  |  | 59 |  |  |  |  |  | 85 |  |  |  |  |  |  |
| 22 |  |  |  |  |  | 60 |  |  |  |  |  | 86 |  |  |  |  |  |  |
| 23 |  |  |  |  |  | 61 |  |  |  |  |  | 87 |  |  |  |  |  |  |
| 24 |  |  |  |  |  | 62 |  |  |  |  |  | 88 |  |  |  |  |  |  |
| 25 |  |  |  |  |  | 63 |  |  |  |  |  | 89 |  |  |  |  |  |  |
| 26 |  |  |  |  |  | 64 |  |  |  |  |  | 90 |  |  |  |  |  |  |
| 27 |  |  |  |  |  | 65 |  |  |  |  |  | 91 |  |  |  |  |  |  |
| 28 |  |  |  |  |  | 66 |  |  |  |  |  | 92 |  |  |  |  |  |  |
| 29 |  |  |  |  |  | 67 |  |  |  |  |  | 93 |  |  |  |  |  |  |
| 30 |  |  |  |  |  | 68 |  |  |  |  |  | 94 |  |  |  |  |  |  |
| 31 |  |  |  |  |  | 69 |  |  |  |  |  | 95 |  |  |  |  |  |  |
| 32 |  |  |  |  |  | 70 |  |  |  |  |  | 96 |  |  |  |  |  |  |
| 33 |  |  |  |  |  | 71 |  |  |  |  |  | 97 |  |  |  |  |  |  |
| 34 |  |  |  |  |  | 72 |  |  |  |  |  | 98 |  |  |  |  |  |  |
| 35 |  |  |  |  |  | 73 |  |  |  |  |  | 99 |  |  |  |  |  |  |
| 36 |  |  |  |  |  | 74 |  |  |  |  |  | 100 |  |  |  |  |  |  |
| 37 |  |  |  |  |  | 75 |  |  |  |  |  | 102 |  |  |  |  |  |  |
| 38 |  |  |  |  |  | 76 |  |  |  |  |  | 102 |  |  |  |  |  |  |

Table of Contents

1.	GENERAL	4

1.1	Scope	4

1.2	Project data	4

1.3	Language and Measurement System	4

2.	REFERENCE DOCUMENTS	5

3.	UNIT IDENTIFICATION	5

3.1	Well Head Facilities	5

3.2	Production Unit	5

4.	EQUIPMENT NUMBERING SYSTEM	6

4.1	Main Equipment Unit	6

4.2	Drivers for Main Equipment	6

4.3	Package Units	6

5.	INSTRUMENT EQUIPMENT NUMBERING SYSTEM	7

6.	PIPING LINE NUMBERING SYSTEM	7

7.	ELECTRICAL EQUIPMENT NUMBERING SYSTEM	7

7.1	Electrical Equipment Numbering System	7

7.2	Electrical Cable Numbering System	7

8.	PIPE RACK AND SLEEPER NUMBERING SYSTEM	8

9.	Attachments	8

9.1	Attachment #1: Equipment Code (1 Sheets)	8

9.2	Attachment #2: Driver Code (1 Sheets)	11

9.3	Attachment #3: Instrument Code (4 Sheets)	13

9.4	Attachment #4: Line Service Code (1 Sheets)	17

9.5	Attachment #5: Electrical Equipment Code (1 Sheets)	19

# GENERAL

## Scope

The petroleum engineering and development department of national Iranian oil company (PEDEC) has planned to develop the green Band-e-Karkheh oil field. The Band-e-Karkheh Field (BKH) is part of the Mehr Block which lies in the main producing area of the Central Zagros fold belt. The BKH field is located north-west of Ahwaz in Khuzestan province, onshore western Iran. The reservoir is an elongate anticline structure, immediately adjacent to the giant Ahwaz Field, which is less than 15km to the southeast.

Because of lack of data, and production ambiguities, the field development is defined in 2 phases:

Phase A (Early production phase) including:

- Drilling & repair of 3 oil producing wells (BKH_2, 5 and 7).

- EPC of one Gathering center and first stage separation with crude transportation pipeline to Ahwaz- 3 production unit for more treatment.

- Preparation of MDP for phase B

Phase B named as phase 1 (Fully development phase) including:

- Drilling of more new wells (17 oil producing and 1  water disposal)

- EPC of oil production, desalination, stabilization, sweetening and gas re-pressurizing & transportation facilities

The phase A basic design and EPC contractor tendering were finished and is under EPC activities execution.

This document is prepared by Pars Consulting Engineers (PCE) according to basic design of Phase 1 (Phase B) facilities which is awarded to this company by PEDEC.

## Project data

Project name: Band-E-Karkheh Early Production Facilities

Client: National Iranian Oil Company (NIOC)

Site location: north-west of Ahwaz in Khuzestan province

Contract number: 88/603

## Language and Measurement System

All engineering and design data including technical information of all quotation shall be in English with consistent use of metric units of measurement. All drawings, instruction for installation, operating manuals, maintenance manuals, and vendor and sub-vendors documents shall be in English language.

# REFERENCE DOCUMENTS

In the procedure of defining this numbering regulation, the following standards have been referenced as the basis:

- IPS (IRANIAN PETROLEUM STANDARDS)

- ISA (INSTRUMENT SOCIETY OF AMERICA)

# UNIT IDENTIFICATION

Based on client’s requirement units in this project will be defined as below:

## Well Head Facilities

This process including of several wells with appropriated devices which is called production wells unit with tag no.  WXX (XX=01~99)

## Production Unit

Main Units:

Main unit which is called process unit and will be described as follow:

Unit 10

Including:

- Main Flow Line and Manifold

- Separation Area Including 1st & 2nd Separators & Test Separator

- Crude Desalting Package

- Crude Oil Stabilization Package

- Storage tanks

- Crude Oil Metering Package

- Main Pipeline and Launcher/Receiver Trap

- Hot oil package

- Gas Re-Pressurizing & Transportation Facilities

- Main Pipeline and Launcher/Receiver Trap

- Export Gas Metering Package Including Prover

- Fuel gas supply system

- chemical packages in Process plant

Off site utility facilities including of:

Unit 20

- Instrument air package

- Nitrogen package

Unit 30

- Raw Water / Potable Water supply &Treatment Package

Unit 40

- Produced Water Treatment Package

Unit 50

- Flare & Burn Pit Package

Unit 60

- Drainage System (Open & Close Drain System)

- Evaporation Pond

# EQUIPMENT NUMBERING SYSTEM

## Main Equipment Unit

Main Equipment shall be numbered in the following manner.

Main Equipment Numbering System

X – XX X – X

A     B  C    D

| A |  | Equipment code (As per attachment 1) |
| --- | --- | --- |
| B |  | Unit key number (2 letters ) |
| C |  | Sequence number 0 through 9 |
| D |  | A letter suffix to represent Main/Standby equipment(From A to Z for duplicate/triplicate, ETC . . .) |

## Drivers for Main Equipment

Drivers for main equipment shall be numbered as follows:

Drivers Numbering System

X – XX X – X – XXX

A     B  C    D      E

| A |  | Equipment code (As per attachment 1) |
| --- | --- | --- |
| B |  | Unit key number (2 letters) |
| C |  | Sequence number 0 through 9 |
| D |  | A letter suffix to represent Main/Standby equipment |
|  |  | (From A to Z for duplicate/triplicate, ETC . . .) |
| E |  | Driver Code (As per attachment 2) |

## Package Units

Package units shall be numbered as follows:

Packages for well head:

PK –W XX X

A  B

| A |  | Unit key number (2 letters - (XX=01~99)) |
| --- | --- | --- |
| B |  | Sequence number 0 through 9 |

Packages for Process:

PK – XXX

A

| A |  | Sequence number 0 through 999 |
| --- | --- | --- |

# INSTRUMENT EQUIPMENT NUMBERING SYSTEM

Instrument Equipment shall be numbered as follows:

For well head:

WXX – XXXX – XX XX

A        B         C   D

| A |  | Unit key number (2 letters - (XX=01~99)) |
| --- | --- | --- |
| B |  | Functional Identification of the Instrument Equipment |
| C |  | Page Number of P&ID |
| D |  | Sequence number 00 through 99 |

For Process:

XX –X XXX – XX XX

A        B         C   D

| A |  | Unit key number (2 letters) |
| --- | --- | --- |
| B |  | Functional Identification of the Instrument Equipment |
| C |  | Page Number of P&ID |
| D |  | Sequence number 00 through 99 |

# PIPING LINE NUMBERING SYSTEM

Piping line shall be numbered as follows:

Piping line Numbering System

XXX – XX – XXX – XXXX – X” – XX

A        B        C          D       E        F

| A |  | Line service code, two or three letter code (as per attachment 4) |
| --- | --- | --- |
| B |  | Unit key number (2 letters for process unit; WXX(XX=01~99)for well head) |
| C |  | Serial line number 001 to 999. Last character is to distinguish similar |
|  |  | lines in similar trains |
| D |  | Represents the piping line class, according to project piping material |
|  |  | specification |
| E |  | Line size in inches |
| F |  | Insulation/tracing/above or underground symbol |

Note: For detail information refer to P&ID for Symbols and Legends,

# ELECTRICAL EQUIPMENT NUMBERING SYSTEM

## Electrical Equipment Numbering System

Electrical Equipment Numbering System

XXX – XX

A        B

| A |  | Electrical Equipment code (As per attachment 5) |
| --- | --- | --- |
| B |  | Sequence number 01 through 99 |

## Electrical Cable Numbering System

Electrical Cable Numbering System

aaa – bbb – X X – X

A B    C

aaa: This section represents start point of electrical cable; it may be electrical or mechanical equipment.

bbb: This section represents end point of electrical cable, it may be electrical or mechanical equipment.

Numbering system specified at section 4.1 (X – XX X – X) and 7.1 (XXX – XX).

| A |  | Electrical Cable type (As per attachment 5) |
| --- | --- | --- |
| B |  | Sequence number of core |
| C |  | A letter suffix to represent multiple cables to one equipment |
|  |  | (From A to Z) |

# PIPE RACK AND SLEEPER NUMBERING SYSTEM

Pipe rack and sleeper shall be numbered as follows:

Pipe rack and sleeper Numbering System

XX – XX – X

A      B     C

| A |  | PR for Pipe rack and SLP for sleeper |
| --- | --- | --- |
| B |  | Unit key number (2 letters) |
| C |  | Sequence number 01 through 99 |

Note: numbering of package equipment shall be finalized by vendor.

# Attachments

## Attachment #1: Equipment Code (1 Sheets)

Refer to page: 11

## Attachment #2: Driver Code (1 Sheets)

Refer to page: 13

## Attachment #3: Instrument Code (4 Sheets)

Refer to page: 15

## Attachment #4: Line Service Code (1 Sheets)

Refer to page: 20

## Attachment #5: Electrical Equipment Code (1 Sheets)

Refer to page: 22

Attachment # 1

Equipment Code

(1 Sheets)

| Equipment |  | Abbreviation |
| --- | --- | --- |
| Air Cooler |  | AC |
| Air Foam Chamber Blower Compressor |  | FC BL C |
| Deluge valve K.O Drum Desalter |  | DV D DE |
| Exchanger |  | E |
| Filter |  | F |
| Fixed Water Hydrants |  | FWH |
| Fixed Water Monitors |  | FWM |
| Fire Water Hydrants Fire water Hose Reel |  | FH HR |
| Fire Water Monitors Foam Package System Direct Furnace & Fire heater Hose box Membrane |  | FM FP FU HB M |
| Portable Foam/Water Monitor |  | PM |
| Pulsation Dampener |  | PD |
| Pump |  | P |
| Scrubber Scraper Trap (Launcher) Scraper Trap (Receiver) Separator |  | SC LA RE SP |
| Strainer Static Mixer Tower |  | ST MX TW |
| Tank; API, Silo, Hopper Turbo Generator |  | TK TG |
| Vessel |  | V |

Attachment # 2

Driver Code

(1 Sheets)

| Driver |  | Abbreviation |
| --- | --- | --- |
| Diesel Engine |  | DE |
| Gas Engine |  | GE |
| Gas Turbine |  | GT |
| Hydraulic Turbine |  | HT |
| Electric Motor |  | M |
| Turbo Expander |  | TEX |

Attachment # 3

Instrument Code

(4 Sheets)

- Instrument Cabinet Codes

| Instrument Cabinet |  | Abbreviation |
| --- | --- | --- |
| Panel |  | PNL |
| Unit Control Panel |  | UCP |
| System cabinet |  | SYS |
| Marshaling cabinet |  | MAR |
| Interposing relay panel |  | IRP |
| Operation work station |  | OWS |
| Engineering work station |  | EWS |
| Printer |  | PRT |

- Instrument Function Codes

| Function | Abbreviation |
| --- | --- |
| Alarm, Indicator | XA,XI |
| Blow Down Valve | BDV |
| Blow Down Valve Position Switch Close | ZSC |
| Blow Down Valve Position Switch Close Indicator | ZLC |
| Blow Down Valve Position Switch Open | ZSO |
| Blow Down Valve Position Switch Open Indicator | ZLO |
| Control Valve | F/L/PV |
| Control Valve Position Switch Close | F/L/ZSC |
| Control Valve Position Switch Open | F/L/ZLO |
| Corrosion Coupon | CC |
| Corrosion Probe | CP |
| Differential Pressure Gauge | DPG |
| Differential Pressure Indicator | DPI |
| Differential Pressure Transmitter | DPT |
| Emergency Shutdown Valve | ESDV |
| Emergency Shutdown Valve Position Switch Close | ZSC |
| Emergency Shutdown Valve Position Switch Close Indicator | ZLC |
| Emergency Shutdown Valve Position Switch Open | ZSO |
| Emergency Shutdown Valve Position Switch Open Indicator | ZLO |
| Emergency Stop (From ESD) | MSL |
| Fault | MXA |
| Flow Computer | FQY |
| Flow Element | FE |
| Flow Function | FY |
| Flow Gauge | FG |
| Flow Indicator | FI |
| Flow Indicator/Controller | FIC |
| Flow Indicator/Totalizer | FQI |
| Flow Transmitter | FT |
| Hand Switch | HS |
| Level Alarm High High | LAHH |
| Level Gauge | LG |
| Level Indicator | LI |
| Level Indicator/Alarm | LIA |
| Level Indicator/Controller | LIC |
| Level Transmitter | LT |
| Motor Operated Valve | MOV |
| Motor Operated Valve Position Switch Close | ZSC |
| Motor Operated Valve Position Switch Close Indicator | ZLC |
| Motor Operated Valve Position Switch Open | ZSO |
| Motor Operated Valve Position Switch Open Indicator | ZLO |
| Pressure Alarm High | PAH |
| Pressure Alarm Low | PAL |
| Pressure Alarm Low Low | PALL |
| Pressure Function | PY |
| Pressure Gauge | PG |
| Pressure Indicator | PI |
| Pressure Indicator/Controller | PIC |
| Pressure Safety Valve | PSV |
| Pressure Transmitter | PT |
| Remote Push Button Start | MSP |
| Remote Push Button Stop | MST |
| Remote/Local Selection | MXLR |
| Restriction Orifice | RO |
| Run/Stop Status | MXI |
| Temperature Gauge | TG |
| Temperature Indicator | TI |
| Temperature Indicator/Controller | TIC |
| Temperature Transmitter | TT |

- Fire and Gas Function Codes

| Function | Abbreviation |
| --- | --- |
| Beacon light | BL |
| Beam gas detector | BGD |
| Beam smoke detector | BSD |
| CO2 release | CR |
| ESD level 1 | ESD-1 |
| ESD level 2 | ESD-2 |
| ESD level 3 | ESD-3 |
| F&G local panel | FLP |
| Gas detector | GD |
| Heat detector | HD |
| Horn | HO |
| HVAC trip | HVAC |
| IR Flame detector | FD |
| Linear Heat detector | LHD |
| Manual alarm call point | MAC |
| Mini Horn | MHO |
| Photoelectric smoke detector | SD |
| Pressure switch high | PSH |
| Toxic gas detector | TGD |

Attachment # 4

Line Service Code

(1 Sheets)

| Service |  |  | Abbreviation |  |
| --- | --- | --- | --- | --- |
| AIR |  |  |  | AIR |
| CHEMICAL DRAIN |  |  |  | CD |
| CHEMICAL ADDITIVE |  |  |  | CHM |
| COLD HYDROCARBON DRAIN |  |  |  | CHD |
| CRUDE OIL |  |  |  | CRD |
| FIRE FOAM |  |  |  | FF |
| FIRE WATER |  |  |  | FW |
| FUEL GAS |  |  |  | FLG |
| HYDROCARBON GAS |  |  |  | GAS |
| GAS OIL HOT OIL |  |  |  | GO HOL |
| HYDROCARBON LIQUID |  |  |  | HC |
| HYDROCARBON DRAIN |  |  |  | HD |
| HIGH PRESSURE FLARE |  |  |  | HFL |
| INSTRUMENT AIR |  |  |  | IA |
| LOW PRESSURE FLARE |  |  |  | LFL |
| LUBRICANT OIL |  |  |  | LO |
| NITROGEN |  |  |  | N2 |
| OILY WATER DRAIN |  |  |  | OWD |
| PLANT AIR |  |  |  | PA |
| PURGE GAS |  |  |  | PG |
| PROCESS GAS |  |  |  | PRG |
| POTABLE WATER |  |  |  | PW |
| PROCESS WATER |  |  |  | PWA |
| RAW WATER |  |  |  | RW |
| SERVICE WATER |  |  |  | SW |

Attachment # 5

Electrical Equipment Code

(1 Sheets)

| Electrical |  |  |
| --- | --- | --- |
| Equipment |  | Abbreviation |
| Transformer |  | TRS |
| 20 kV, 10 kV & 6 kV Switchgear |  | SWG |
| 400 V Motor Control Center |  | MCC |
| Distribution Panel Board |  | LDP |
| Grounding Resistor |  | GRS |
| Uninterruptible Power Supply System |  | UPS |
| Direct Current Panel |  | DCS |
| 400 V Emergency Panel |  | LEP |
| Power Factor Correction |  | PFC |